RDPMS

CAN Bus Analyzer & Serial Monitor

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

Project Report

| Document Type | Technical Project Report |
| --- | --- |
| Version | 1.0.0 |
| Date | 6 July 2026 |
| Platform | Windows x64 Desktop Application |
| Technology | Flutter (Dart) |

© 2026 RDPMS — Confidential

# Table of Contents

| 1. | Project Overview | 3 |
| --- | --- | --- |
| 2. | Architecture & Code Structure | 4 |
| 3. | Complete Feature List | 5 |
| 3.1 | Core CAN Bus Communication | 5 |
| 3.2 | User Interface Features | 6 |
| 3.3 | Send Sequence System | 7 |
| 3.4 | Firmware Upload — Single Device OTA | 8 |
| 3.5 | Firmware Upload — Bulk OTA (Multi-Device) | 9 |
| 3.6 | Design & Polish | 10 |
| 4. | CAN Protocol Implementation Details | 11 |
| 4.1 | Frame Formats | 11 |
| 4.2 | CAN FD DLC Mapping | 12 |
| 4.3 | Board Types (OTA) | 12 |
| 5. | Technology Stack | 13 |
| 6. | Key Architectural Decisions | 14 |

# 1. Project Overview

RDPMS-COM-READER is a professional-grade CAN Bus Analyzer and Serial Monitor Windows desktop application designed for real-time communication with CAN-enabled hardware devices over USB.

| Item | Detail |
| --- | --- |
| Application Type | Windows Desktop Application |
| Technology | Flutter (Dart) |
| Purpose | CAN Bus monitoring, frame analysis, data logging, and firmware OTA updates |

The application operates in fully native USB mode — all CAN frame parsing, building, and serial I/O happens natively in Dart using flutter_libserialport. No external server or network connection is required.

# 2. Architecture & Code Structure

Project Structure — Flutter Desktop App (serial_monitor/)

| File Path | Description |
| --- | --- |
| main.dart | App entry point, Provider setup, theme initialization |
| core/config/app_constants.dart | Global constants (baud rates, display modes, UI limits) |
| core/config/can_config.dart | CAN bus configuration model & CONNECT frame builder |
| core/config/models.dart | Data models (SerialMessage, SerialTab, SendSequence, etc.) |
| core/controllers/port_controller.dart | Main state management — tabs, connection, CAN frames (888 lines) |
| core/routes/app_routes.dart | Route definitions (splash → home) |
| core/services/serial_port_service.dart | Native USB serial port I/O (404 lines) |
| core/services/frame_builder.dart | Binary CAN TX frame builder (F1 01 protocol) |
| core/services/frame_parser.dart | Stateful CAN RX frame parser (A1, F1, D1 protocols) |
| core/services/firmware_upload_service.dart | Single-device OTA firmware upload (477 lines) |
| core/services/bulk_firmware_service.dart | Multi-device broadcast OTA upload (1,175 lines) |
| core/view/splash_screen.dart | Animated branded splash screen |
| feature/.../serial_port_screen.dart | Main screen — sidebar, toolbar, CAN config (2,352 lines) |
| feature/.../tab_view.dart | Communication console & data table (549 lines) |
| feature/.../firmware_upload_dialog.dart | Single-device firmware upload UI (824 lines) |
| feature/.../bulk_firmware_dialog.dart | Multi-device bulk OTA UI (1,096 lines) |
| utils/theme/app_theme.dart | Complete design system — 40+ color tokens (245 lines) |
| utils/widgets/message_widget.dart | CAN data table row widget |
| utils/widgets/tab_bar_widget.dart | Custom renameable tab bar with CAN ID filtering |

# 3. Complete Feature List

The RDPMS CAN Bus Analyzer includes 70 fully implemented features across 6 categories.

## 3.1 Core CAN Bus Communication

| # | Feature | Status | Details |
| --- | --- | --- | --- |
| 1 | USB Serial Port Connection | ✅ Done | Native USB I/O via flutter_libserialport, auto-detects COM ports |
| 2 | CAN Connect Handshake (0xA0 → 0xA1) | ✅ Done | Sends 7-byte CONNECT frame, waits for 5-byte ACK with 3s timeout |
| 3 | Classic CAN Support | ✅ Done | Normal, Loopback, and Silent modes |
| 4 | CAN FD Support | ✅ Done | Data baud rates: 1/2/4/5/8 Mbps, BRS and ISO/Non-ISO flags |
| 5 | Dual Channel Support | ✅ Done | Channel 1 and Channel 2 selectable |
| 6 | Real-time CAN Frame Reception | ✅ Done | Stateful binary parser: timestamp, CAN ID, DLC, channel, data |
| 7 | CAN Frame Transmission | ✅ Done | Build and send TX frames (F1 01) with Standard/Extended IDs |
| 8 | Baud Rate Configuration | ✅ Done | 10 kbps to 1 Mbps (10 options) for nominal rate |
| 9 | Auto Port Detection & Hot-Plug | ✅ Done | Polls every 1.5s, detects plug/unplug, auto-disconnects |
| 10 | CAN Configuration Persistence | ✅ Done | All CAN settings saved to SharedPreferences |

## 3.2 User Interface Features

| # | Feature | Status | Details |
| --- | --- | --- | --- |
| 11 | Multi-Tab Interface | ✅ Done | Up to 20 tabs, each with independent message buffer (5,000 max) |
| 12 | Tab CAN ID Filtering | ✅ Done | Each tab assigned a CAN ID — only matching frames appear |
| 13 | Renameable Tabs | ✅ Done | Double-click to rename, drag-and-drop reorder |
| 14 | Tab Persistence | ✅ Done | Tabs, names, CAN IDs, send sequences saved across sessions |
| 15 | Professional CAN Data Table | ✅ Done | 10-column table: Index, Time, Timestamp, Channel, Direction, ID, Type, Format, DLC, Data |
| 16 | Multi-Format Display | ✅ Done | Toggle between ASCII, HEX, Decimal, Binary display modes |
| 17 | Direction Color Coding | ✅ Done | Red for TX (sent), Green for RX (received) with indicator bars |
| 18 | Auto-Scroll | ✅ Done | Auto-scrolls to latest message, pauses when user scrolls up |
| 19 | Message Search/Filter | ✅ Done | Search by CAN ID, direction, data content, type |
| 20 | Right-Click Context Menu | ✅ Done | Copy Row, Copy Data, Copy CAN ID, Filter by ID |
| 21 | Keyboard Shortcuts | ✅ Done | Ctrl+N (new tab), Ctrl+L (clear), Ctrl+E (export) |
| 22 | CSV Export | ✅ Done | Export to Desktop with timestamped filename |
| 23 | TX/RX Byte Counters | ✅ Done | Live byte count displayed in control strip |
| 24 | Dynamic Window Title | ✅ Done | Shows "RDPMS Serial Monitor — COM4 Connected" or "Disconnected" |
| 25 | Error Notification System | ✅ Done | Snackbar notifications with error log (max 100 entries) |
| 26 | About Dialog | ✅ Done | App info, version, keyboard shortcuts reference |
| 27 | Animated Splash Screen | ✅ Done | Branded RDPMS splash with fade, scale, pulse animations |
| 28 | Resizable Sidebar | ✅ Done | Drag divider to resize send sequences panel (180–500px) |

## 3.3 Send Sequence System

| # | Feature | Status | Details |
| --- | --- | --- | --- |
| 29 | Send Sequence Table | ✅ Done | Named, saveable CAN message presets per tab |
| 30 | CAN Frame Builder UI | ✅ Done | Configure CAN ID, Frame Format, Frame Type, Channel |
| 31 | Multi-Format Input | ✅ Done | Enter data in HEX, ASCII, Decimal, or Binary with live conversion |
| 32 | Repeat Count | ✅ Done | Send a sequence N times |
| 33 | Send Cycle Delay | ✅ Done | Configurable inter-message delay in milliseconds |
| 34 | ID Increment Mode | ✅ Done | Auto-increment CAN ID with each repeat (wraps at limits) |
| 35 | Data Increment Mode | ✅ Done | Auto-increment data bytes with carry propagation |
| 36 | Sequence Documentation | ✅ Done | Add notes/documentation per sequence |
| 37 | Trailing Placeholder | ✅ Done | Always shows an empty row to add new sequences quickly |

## 3.4 Firmware Upload — Single Device OTA

| # | Feature | Status | Details |
| --- | --- | --- | --- |
| 38 | File Selection | ✅ Done | Windows native file picker (PowerShell) for .bin files |
| 39 | CRC-16/Modbus Calculation | ✅ Done | Whole-file CRC and per-frame CRC calculation |
| 40 | OTA Header (0x02) | ✅ Done | Sends board type, bin size, frame count, file CRC |
| 41 | Binary Data Frames (0x42 0x49) | ✅ Done | 64-byte CAN FD frames with 60-byte payload + CRC |
| 42 | Completion Signal (0x4F 0x4B) | ✅ Done | End-of-transmission frame |
| 43 | ACK/NACK Handling | ✅ Done | ACK (0x79), NACK codes (0xE1–0xE7) with descriptions |
| 44 | Board Type Selection | ✅ Done | 8 board types: AC Voltage, AC Current, DC voltages, etc. |
| 45 | Progress Bar & Percentage | ✅ Done | Real-time progress with frame count display |
| 46 | Transfer Log Console | ✅ Done | Dark-themed terminal with color-coded TX/RX/error entries |
| 47 | Cancel Upload | ✅ Done | Cancel mid-upload with proper cleanup |
| 48 | Inter-frame Delay | ✅ Done | Configurable delay between data frames |
| 49 | Info Cards | ✅ Done | File size, frame count, CRC-16, chunk size display |
| 50 | Console Suppression | ✅ Done | Main console suppressed during upload to prevent flooding |

## 3.5 Firmware Upload — Bulk OTA (Multi-Device Broadcast)

| # | Feature | Status | Details |
| --- | --- | --- | --- |
| 51 | CAN Bus Scan (0x01) | ✅ Done | Firmware version request broadcast; discovers all nodes |
| 52 | Board Discovery Table | ✅ Done | Shows CAN ID, Device ID, Board Type, Version, Status |
| 53 | Board Selection | ✅ Done | Checkbox selection of which boards to update |
| 54 | Board Type Filter | ✅ Done | Filter scan by board type (All, AC Voltage, etc.) |
| 55 | Broadcast Header (0x02) | ✅ Done | Single header sent to all boards simultaneously |
| 56 | Per-Board ACK Collection | ✅ Done | Collects staggered ACKs with grace timeout |
| 57 | Broadcast Data Frames | ✅ Done | 64-byte CAN FD frames sent to all boards in parallel |
| 58 | Completion with Retry | ✅ Done | Sends completion signal up to 3 times |
| 59 | Per-Board Status Tracking | ✅ Done | Real-time per-board progress, status, version display |
| 60 | New Version Reporting | ✅ Done | Shows new firmware version per board after OTA |
| 61 | Force Application Jump (0x41) | ✅ Done | Clears OTA flag and boots into application |
| 62 | Boot Status Codes | ✅ Done | Parses 0xA0–0xA3 boot status codes |
| 63 | Error Codes (0xE1–0xE7) | ✅ Done | Missing frames, CRC error, Board mismatch, etc. |
| 64 | Manual Mode | ✅ Done | Step-by-step frame sending for debugging |
| 65 | Mock Mode | ✅ Done | Full simulation without hardware |
| 66 | Mixed Board Type Detection | ✅ Done | Prevents broadcast to mixed board types |

## 3.6 Design & Polish

| # | Feature | Status | Details |
| --- | --- | --- | --- |
| 67 | Professional Light Theme | ✅ Done | Inspired by PCAN-View, Vector CANalyzer, BusMaster |
| 68 | Complete Design System | ✅ Done | 40+ color tokens, reusable decorations, Google Fonts |
| 69 | Windows Installer | ✅ Done | Inno Setup script with desktop shortcut, start menu, auto-launch |
| 70 | Release Package | ✅ Done | Pre-built release zip available |

# 4. CAN Protocol Implementation Details

## 4.1 Frame Formats

| Frame | Byte Pattern | Direction | Description |
| --- | --- | --- | --- |
| CONNECT | A0 01 [ch] [baud] [mode] [flags] 00 | PC → Device | 7-byte CAN connection request |
| CONNECT ACK | A1 [status] [ch] [canType] [rsv] | Device → PC | 5-byte connection response |
| TX Frame | F1 01 [canId 4B LE] [dlc+ch] [data…] | PC → Device | Variable-length CAN TX frame |
| RX Frame | F1 00 [ts 4B] [canId 4B] [dlc+ch] [data…] | Device → PC | Variable-length CAN RX frame |
| Heartbeat | D0 00 | PC → Device | 2-byte keepalive request |
| Heartbeat ACK | D1 [status] | Device → PC | 2-byte keepalive response |
| OTA Header | 02 [type] [size 2B] [frames 2B] [crc 2B] | PC → Device | 8-byte firmware header |
| OTA Data | 42 49 [60B data] [crc 2B] | PC → Device | 64-byte firmware data frame |
| OTA Complete | 4F 4B [62B padding] | PC → Device | 64-byte end-of-transmission |
| OTA ACK | 79 … | Device → PC | Success acknowledgement |
| OTA NACK | E1–E7 … | Device → PC | Error codes with meanings |
| Boot Status | A0–A3 | Device → PC | Power-on boot status codes |
| Force Jump | 41 80 80 | PC → Device | Force application jump command |
| Force Jump ACK | B0 | Device → PC | Jump accepted response |
| Scan Request | 01 01 [boardType] | PC → Device | Firmware version discovery |

## 4.2 CAN FD DLC Mapping

| DLC Code | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Length | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 12 | 16 | 20 | 24 | 32 | 48 | 64 |

## 4.3 Board Types (OTA)

| Code | Abbreviation | Label |
| --- | --- | --- |
| 0x00 | ALL | All Boards |
| 0x01 | AV | AC Voltage |
| 0x02 | AC | AC Current |
| 0x03 | DH | DC High Voltage |
| 0x04 | DL | DC Low Voltage |
| 0x05 | HI | DC High Current |
| 0x06 | LI | DC Low Current |
| 0x07 | AM | Accelerometer |
| 0x08 | DC | Digital |

# 5. Technology Stack

| Layer | Technology | Version |
| --- | --- | --- |
| Framework | Flutter | SDK ≥3.10.0 |
| Language | Dart | 3.10+ |
| State Management | Provider | 6.1.2 |
| Serial Port I/O | flutter_libserialport | 0.4.0 |
| Local Storage | shared_preferences | 2.3.3 |
| Typography | google_fonts | 6.2.1 |
| Installer | Inno Setup | — |
| Target OS | Windows x64 | — |

# 6. Key Architectural Decisions

1. Fully Native USB Architecture

The application performs all CAN protocol operations natively using flutter_libserialport — no external server or network dependency is required.

2. Stateful Binary Parser

The CanFrameParser class accumulates raw USB bytes in a buffer and extracts complete frames only when enough bytes are available, correctly handling partial reads and frame boundaries.

3. Multi-Tab CAN ID Isolation

Each tab can be assigned a CAN ID filter, allowing the user to monitor specific nodes while all raw data still flows through the system.

4. Broadcast OTA Protocol

The bulk firmware system sends a single header + data stream to all boards simultaneously, then collects per-board ACKs with staggered timeout windows and retry logic.

5. Console Suppression During OTA

During firmware uploads, the main console RX/TX callbacks are suppressed to prevent thousands of data frames from flooding the UI message table.